Circuit Analysis (Lab3)

Experiment 3:  Introduction to Multimeter and Basic Series Circuit Implementation

Name: Sania Khan

Registration No: 14

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeters, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

________________________________________________________________________________ Objectives: To get understanding of how to use a multimeter and make measurements in your own constructed series circuits.

Equipment required:

Power Supply, Breadboard, Connecting wires, Resistances, Multimeter.

What is a multimeter?

A multimeter is a measurement tool necessary in electronics. It combines three essential features: a voltmeter, ohmmeter, and ammeter, and in some cases continuity. A multimeter allows you to understand what is going on in your circuits. Whenever something in your circuit isn’t working, the multimeter will help you troubleshooting.

Parts of Multimeter:

Display: this is where the measurements are displayed

Selection knob: this selects what you want to measure

Ports: this is where you plug in the probes

Probes: a multimeter comes with two probes. Generally, one is red and the other is black. Note: There isn’t any difference between the red and the black probes, just the color. So, assuming the convention:

- The black probe is always connected to the COM port.

- The red probe is connected to one of the other ports depending on what you want to measure.

# Ports

The “COM” or “–“ port is where the black probe should be connected. The COM probe is conventionally black.

- A is used when measuring large currents, greater than 200mA

- mA is used to measure current

- VΩ allows you to measure voltage and resistance and test continuity.

Figure 1 Digital Multimeter (DMM)

# Introduction to resistance measurements

Resistance is measured with the circuit’s power turned off. The ohmmeter sends its own current through the unknown resistance and then measures that current to provide a resistance value readout.

Resistance measurement procedures

Follow the steps below to measure resistance:

- Turn off the power to the circuit. Remove or isolate the component to be tested.

- Plug the test probes into the appropriate probe jacks. Note that the jacks used may be the same ones used to measure volts.

- Select the ohms function by turning the function switch to ohms. Start with the lowest setting.

- Touch the probes together to check the leads, connections and battery life. The meter should display zero or a very small amount of resistance for the test leads. With the leads apart, the meter should display OL or I, depending on the manufacturer.

- Connect the tips of the probes across the break in the component or portion of the circuit for which you want to determine resistance. If you get an OL (over limit), move to the next level.

- View the reading on the display unit. Be sure to note the unit of measurement.

- After all the resistance readings are complete, turn the DMM off to prevent the battery from draining

Figure 2 Using a DMM to measure resistance

# Introduction to voltage measurements

Voltage measurements are very easy to make with a DMM. On meters with manual range selection, start with a value one setting higher than expected. An auto range DMM will automatically select the range based on the voltage present. Figure 4 shows the process.

Note:

1/ 1000 V=1mV

1000 V=1kV

Follow these steps to measure voltage:

- Select the DC or AC volts function by turning the function switch to DC or AC volts.

- Plug the test probes into the appropriate probe jacks.

- Connect the tips of the probes across the source or load.

- View the reading on the display unit. Be sure to note the unit of measurement. If you are testing DC voltage and a negative sign appears in the display, the polarity of your probes is incorrect and needs to be reversed.

Figure 3 Using a DMM to measure voltage

# Introduction to current measurements

As you have seen, the placement of meter leads for voltage measurements is straightforward. The leads are simply connected across, or in parallel with, the points of voltage to be measured. For current measurements, however, the process is slightly more complex.

First, the circuit must be opened at the test points and the meter inserted in series at that opening. The total current must flow through the meter. To allow the measurement to be made without disturbing the circuit itself, the current meter must have very little internal resistance.

If the meter is inadvertently connected across a P.D. (potential difference) or in parallel with a component instead of in series, the small internal resistance will permit a very large current to flow through the meter.

This will most certainly damage the meter severely and perhaps the circuit as well.

On meters with manual range selection, start with the highest current setting and work your way down.

# Current measurement procedures

Follow the steps below to measure current:

- Turn off the power to the circuit that will be measured.

- Open the circuit by disconnecting or unsoldering a connection at a point where you wish to measure current.

- Select the DC or AC amps function by turning the function switch to DC or AC amps.

- Plug the test probes into the appropriate probe jacks. Note that the jacks used may not be the same ones used to measure volts.

- Connect the tips of the probes across the break in the circuit so that the current to be measured flows through the meter. Note that this is a series connection. Never connect the ammeter in parallel with the source or load, as this will cause a short circuit and damage the meter.

- Turn the circuit power back on.

- View the reading on the display unit. Be sure to note the unit of measurement.

- Switch power off and disconnect meter leads from the circuit.

- If testing is finished at this test point, restore the circuit by reclosing the connection. When current measurements are complete, turn the function switch to the OFF position and remove the test leads.

Figure 4 Using a DMM to measure current

Lab Tasks:

  - Measure Current between R1 and R2, and Voltage across R1, R2.

Practical values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.04V | 1.74V | 3.0mA |

Theoretical Values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.03V | 1.96V | 0.595mA |

Circuit 1: Two Resistor Series Circuit (V1, R1, R2)

- Component Selection:

Go to ‘Component Mode’, click the ‘P’ (Pick) button.

Search for and select: RES (Resistor) and VSOURCE (DC Voltage Source).

Place one VSOURCE and two RES components onto the workspace.

- Adding Instruments:

Go to the ‘Instrument’ tab.

Select and place one AC VOLTMETER and one AC AMMETER on the sheet.

- Wiring and Configuration:

Resistors: Connect R1 and R2 in series by drawing a wire between them. Right-click each to change values to 5.1k for R1 and 3.3k for R2.

Voltage Source: Connect the positive terminal of V1 to the start of R1. Set V1 to 5V.

Ground: Go to ‘Terminals Mode’ (often referred to as Subcircuit/Terminal mode), select GROUND, and connect it to the negative terminal of V1.

Ammeter: Connect the AC AMMETER in series between the end of R2 and the ground point.

Voltmeters: Connect one voltmeter in parallel across R1 and another voltmeter in parallel across R2.

  - Measure Current at node 2 and 3 and Voltage across R1, R2, R3.

Practical values

| R1 Voltage | R2 Voltage | R3 voltage | Current at node 1 | Current at node 2 |
| --- | --- | --- | --- | --- |
| 1.5 V | 5V | 2.5V | 0.5mA | 0.5mA |

Theoretical Values:

| R1 Voltage | R2 Voltage | R3 Voltage | Current at node 1 | Current at node 2 |
| --- | --- | --- | --- | --- |
| 1.5 V | 5V | 2.5V | 0.5mA | 0.5mA |

Circuit 2: Three Resistor Series Circuit (B1, R3, R4, R5)

- Component Selection:

In ‘Component Mode’, click the ‘P’ button.

Search for and select: RES (Resistor) and BATTERY (Multi-cell DC source).

Place one BATTERY and three RES components onto the workspace.

- Adding Instruments:

Go to the ‘Instrument’ tab.

Select and place three AC VOLTMETERS and one AC AMMETER.

- Wiring and Configuration:

Resistors: Connect R3, R4, and R5 in a single series line. Set the values to:

R3: 3k

R4: 10k

R5: 5k

Battery: Connect the positive terminal of B1 to the input of R3. Set the battery voltage to 9V.

Ammeter: Connect the AC AMMETER in series, completing the loop from the end of R5 back to the negative terminal of the battery.

Voltmeters: * Connect the first voltmeter in parallel across R3.

Connect the second voltmeter in parallel across R4.

Connect the third voltmeter in parallel across R5.

Post Lab Questions:

- Define energy .

Energy is the ability to do work or cause change. It’s a fundamental concept in science and exists in many different forms.

- Define power.

Power is the rate at which energy is used or work is done.

- What is electric current.

Charge (Electric Charge) is a basic property of matter that causes objects to experience electric forces. It’s what makes electricity possible.

- What is circuit?

An electric circuit is a closed path through which electric current (flow of charge) moves

- What is resistor?

A resistor in a series circuit limits the current, protects components from damage, and helps divide voltage among them.

- What is voltage divider?

The voltage divider rule is a simple way to calculate the voltage across each resistor in a series circuit. It states that the voltage across a resistor is proportional to its resistance compared to the total resistance of the series.

Formula:

Vx=Rx/R(total)×V(total)

- What is series circuit?

A series circuit has all components connected in a single path, so the same current flows through each. Voltage is shared among components, total resistance adds up, and if one component fails, the whole circuit stops. It’s simple but less efficient for equal voltage distribution.

- What is voltmeter?

Voltmeter are designed with high resistance and connected in parallel, while ammeters have low resistance and are connected in series.

- Which combination is used for connection of DMM in measurement of current ?

To measure current, a Digital Multimeter (DMM) is connected in series with the circuit so the current flows through it. To measure voltage, a DMM is connected in parallel across the component so it measures the potential difference without interrupting the circuit.